Unit 103: Electrical scientific principles and technologies

# Worksheet 10: Lighting (learner)

Using your notes, answer the following questions.

1.	The unit and symbol for luminous intensity is the:

2.	The unit and symbol for luminous flux is the:

3.	The unit and symbol for illuminance is the:

4.	With regard to illumination, briefly explain the inverse square law.

5.	A lamp of luminous intensity 500 candela is suspended 2 metres above a work area. Calculate the illuminance directly below the lamp.

6.	The lamp in Question 5 is moved to 3 metres above the work surface. Calculate the illuminance at the work area.

7.	With regard to illumination, briefly explain the cosine law.

8.	A street lighting column suspends a 1500cd light source 6m above the ground. Determine the illuminance 3m to one side of the lamp base.

9.	A warehouse measures 80 metres × 30 metres.

Find the number of lamps required if each lamp has a Lighting Design Lumen (LDL) output of 18 000 lumens. The illumination required for the warehouse area is 400 lux. Utilisation factor = 0.45 and lamp maintenance factor = 0.7.

Incandescent lights

Using your notes, answer the following questions.

10.	Describe what is meant by the term ‘colour rendering’ with regard to lighting and how it is generally expressed.

11.	Describe how the efficiency of lighting sources can be compared with the units used.

12.	What do the initials GLS stand for?

13.	What is the operating position for a GLS lamp?

14.	What is the operating position for a tungsten halogen lamp?

15.	Describe the main safety precautions to be observed with tungsten halogen lamp.

Low pressure mercury

Using your notes, answer the following questions.

16.	The colour rendering of a low pressure mercury lamp is:

17.	The efficacy of a low pressure mercury lamp is:

18.	The average life of in hours of a low pressure mercury lamp is:

19.	The purpose of the ballast or choke in a low pressure mercury lamp is to:

20.	In the switch-start fluorescent fitting, state why a capacitor is placed across the in-coming supply terminals.

21.	The light produced by the discharge in a low pressure mercury lamp is ultra-violet, state how this is converted to visible light.

22.	Draw a fully labelled circuit diagram of a switch (glow) start low pressure mercury fitting (fluorescent).

High-pressure mercury

Using your notes, answer the following questions.

23.	The colour rendering of a high-pressure mercury lamp is:

24.	The colour rendering of a metal halide lamp is:

25.	The efficacy of a high-pressure mercury lamp is:

26.	The efficacy of a metal halide lamp is:

27.	The purpose of the ballast in a high-pressure mercury lamp is to:

28.	In both the high pressure mercury and metal halide lamps, state why argon is in the discharge tube.

29.	High pressure mercury vapour and metal halide lamps are described collectively as HIDs. What does ‘HID’ stand for?

30.	Draw a fully labelled circuit diagram of a high pressure mercury vapour fitting.

Sodium lights

Using your notes, answer the following questions.

31.	The colour rendering of a low pressure sodium lamp is:

32.	The colour rendering of a high pressure sodium lamp is:

33.	The efficacy of a low pressure sodium lamp is:

34.	The efficacy of a high pressure sodium lamp is:

35.	The purpose of the ballast in a high pressure sodium lamp is to:

36.	In the high pressure sodium lamp, state why xenon is in the discharge tube.

37.	Draw a fully labelled circuit diagram of a high-pressure sodium vapour fitting.

LED Lighting

Using your notes, answer the following questions.

38.	The colour rendering of LED lamps is:

39.	The efficacy of LED lamps is:

40.	State the main disadvantage, currently, of LEDs lamps.

41.	State FIVE benefits of LEDs lamps.