Unit 103: Electrical scientific principles and technologies

# Worksheet 11: Principles and applications of electric heating (learner)

Space heating

Using your notes, answer the following questions.

1.	In an under floor heating system what is a cold tail?

2.	State the two main types of heat transference used in space heating.

3.	Research what is meant by the term ‘Economy 7’ and the benefits for the consumer.

Water heating

Using your notes, answer the following questions.

4.	State the two main groups of water heaters.

5.	What temperature should the thermostat be set to in an immersion heater?

6.	State briefly the circuit arrangement for feeding an instantaneous shower.

7.	What device in an instantaneous water heater prevents the user from receiving scalds from over-hot water?

Heating control

Using your notes, answer the following questions.

8.	When setting the temperature of thermostat for water heating the temperature must not be too high or too low. State two factors that directly affect this.

9.	What temperature would it be usual for a room thermostat to be set at?

10.	State briefly the purpose of the programmer.

11.	State the two main types of electrically operated valves used in central heating systems.