Unit 103: Electrical scientific principles and technologies

# Worksheet 3: Basic mechanics (learner)

1.	If a mass of 500kg is raised a distance of 2m. Calculate the amount work done.

2.	A mass of 300kg is raised through a vertical distance of 8m. Calculate the amount work done.

3.	Calculate the work done, when a force of 750N moves an object through a horizontal distance of 15m.

4.	Calculate the work done when a force of 350N moves an object through a distance of 3.5m.

5	If the amount of work done by a force of 100N is 800Nm, calculate the distance the object has moved.

6.	Calculate the distance moved by a trolley of 51kg when the amount of work done is 1020Nm.

7.	Calculate the opposition force of a transformer that moves 40m when the work done is 14 000Nm.

8.	Calculate the weight of a transformer that moves 25m, when the work done is 23 298.75Nm.

9.	An object is moved through a distance of 20m in 5 seconds by a force of 50N. Calculate the power used.

10.	Calculate the power required to move a weight of 300N through a vertical distance of 5m in 2 seconds.

11.	A mass of 40 litres of water is lifted to a height of 10m in 5 seconds. Calculate the power that will be used.

12.	Calculate the power required to raise 2.5m3 of water from a well 12.5m deep in 30 seconds. NB: 1m3 of water = 1000kg.

13.	A hoist raises a load of 3 tonnes through 8 metres in 15 seconds. Determine the power developed by the hoist. NB: 1 Tonne = 1000kg.

14.	An electric motor drives a pump that raises 1 000 litres of water each minute to a tank 20m above ground level. Calculate the power the motor must provide if the pump is only:

a.	50% efficient

b.	80% efficient.

15.	A pump raises 0.15m3 of water per minute from a well 7.5m deep. Calculate the power required to drive the pump if it is only 75% efficient.

16.	An electric motor drives a pump that raises 0.15m3 of water per minute through a vertical height of 35m. Calculate:

a.	the power required

b.	if the pump has an efficiency of 72%, calculate the power required to drive the pump

c.	the line current if the supply voltage to the motor is 220 volts dc.

17.	A pump, which raises 0.12m3 of water per minute from a well 8.5m deep, is driven by a 250 volt dc motor. Assuming the efficiency of the pump is 72% and that of the motor is 78%, calculate the current drawn from the supply by the motor.

18.	A piece of steel wire is to be cut by a pair of wire cutters. The wire is gripped 5mm from the fulcrum whilst the pressure is applied to the handles 80mm from the fulcrum. Calculate the force that must be applied to the handles to cut the wire if it requires a direct cutting force of 160N.

19.	A lever is arranged so that a load of 1000N is to be lifted at a distance of 200mm from the fulcrum. Calculate the force that must be applied 2m from the fulcrum to balance the load.

20.	A crowbar is used to lift a packing case. The load is 200mm from the fulcrum and an effort of 50N is applied 1.25m to the other end of the crowbar. Determine the load in newtons.

21.	A load of 500N is situated on the end of a crowbar 150mm from the fulcrum. An effort of 37.5N is applied to the other end of the crowbar. Determine the length of the crowbar.

22.	The pulley system shown is used to lift a 450kg load. Determine effort in newtons required.

Load

450kg

Effort?