Unit 103: Electrical scientific principles and technologies

# Worksheet 4b: Resistance, resistivity, voltage, current and power (learner)

Resistivity

1.	If a coil of cable 50m long has a conductor resistance of 0.275, calculate the value for 800m of the same cable.

2.	Calculate the resistance of a single core copper cable 200m long and 25mm2 csa. Take the resistivity of copper to be 1.72 x 10-8 ohm/metre.

3.	Determine the resistance of 150m of single copper cable whose csa is 10mm2. Take the resistivity of copper to be 1.72 x 10-8 ohm/metre.

4.	Calculate the resistance of 75m of single aluminium cable with a csa is 4mm2. Take the resistivity of aluminium to be 2.65 x 10-8 ohm/metre.

5.	Find the csa of a single copper cable 500m long, which carries a current of 2.5 amperes and has a volt drop of 3.58 volts. Take the resistivity of copper to be 1.72 x 10-8 ohm/metre.

6.	Calculate the resistance of a 4mm2 csa twin copper cable feeding a motor 150m away. Take the resistivity of copper to be 1.72 x 10-8 ohm/metre.

7.	Calculate the resistance of 100m of the following sizes of twin copper cables. Take the resistivity of copper to be 1.72 x 10-8 ohm/metre:

a.	1mm2

b.	1.5mm2

c.	2.5mm2

d.	4mm2

e.	6mm2

f.	10mm2

8.	Complete the following table:

| Resistance (R) |  | 0.5 | 0.02 |  | 0.172 |
| --- | --- | --- | --- | --- | --- |
| Resistivity () | m | 1.72E-8 |  | 1.72E-8 | 1.72E-8 |
| Length (l) | M |  | 4.23 | 85 | 250 |
| csa (a) | mm2 | 1.0 | 6.0 | 2.5 |  |

9.	A motor takes 45A from a 230v supply. A twin aluminium cable 40m long feeds the motor, each core having a CSA of 16mm2. Calculate the voltage at the motor terminals Take the resistivity of aluminium to be 2.65 x 10-8 ohm/metre.

10.	Two cables have equal resistance but one has a CSA 2.5 times larger than the other. How much longer is the thicker cable than the thinner cable?

11.	An immersion heater takes a current of 13A and is fed through a twin cable, each core having a CSA of 2.5mm2. If the conductors are made of copper and have a resistivity of 1.72 x 10-8 Ω/m, calculate the greatest length of cable which may be used, if the cable voltage drop is not to exceed 11.5 volts.