Unit 103: Electrical scientific principles and technologies

# Worksheet 4e: Resistance, resistivity, voltage, current and power (learner)

### Resistors in parallel

1.	Calculate the total resistance of each branch if the following resistors are connected in parallel:

|  | R1 | R2 | R3 | RT |
| --- | --- | --- | --- | --- |
| a. | 12 | 6 | 4 |  |
| b. | 30 | 90 | 45 |  |
| c. | 120 | 80 | 48 |  |
| d. | 6 | 20 | 30 |  |
| e. | 1.5 | 4 | 12 |  |

2.	Complete the following table assuming that the resistors are connected in parallel:

|  | R1 | R2 | R3 | RT |
| --- | --- | --- | --- | --- |
| a. | 120 |  | - | 48 |
| b. |  | 48 | - | 12 |
| c. |  | 50 | - | 40 |
| d. | 40 |  | 20 | 10 |
| e. | 60 |  | 20 | 10 |

3.	Two resistors of 48 and 80 are connected together in parallel. What would the value of a third resistor have to be, when connected in parallel with the first two, to give a combined resistance total of 15?

4.	A parallel circuit containing three resistors of 1.5, 4 and 12 is connected across a 10 volt supply. Calculate:

a.	the current flowing in each resistor

b.	prove Kirchhoff’s current law.

5.	A parallel circuit containing three resistors of 18, 20 and 30 is connected across a 144 volt supply. Calculate:

a.	the current flowing in each resistor

b.	prove Kirchhoff’s current law.

6.	A parallel circuit containing three resistors of 30, 90 and 45 is connected across a 30-volt supply. Calculate:

a.	the current flowing in each resistor

b.	prove Kirchhoff’s current law.

7.	Three resistors of 0.012, 0.015 & 0.008 respectively are connected in parallel across a 2.4-volt d.c. supply. Calculate:

a.	the current flowing in each resistor

b.	the current in each section of the circuit

c.	the total current drawn from the supply.

8.	Three resistors are connected in parallel across a D.C. supply. The values of two of the resistors are 12 and 18 respectively. If the 18 resistor has a current of 18 amperes flowing through it and the total current drawn from the supply is 81 amperes. Calculate:

a.	the supply voltage

b.	the value of current flowing through each resistor

c.	the value in ohms, of the third resistor.

9.	Four resistors with values of 8, 17, 9 and 6 are connected in series. The 9 resistor has a volt drop of 27 volts across. Calculate (a) the total current, (b) the total resistance (c) the voltage drop across the other three resistors and (d) the supply voltage.

10.	When four resistors are connected in series across 110V supply a current of 2A flows. Three of the resistors have values of 5, 23 and 10. Calculate (a) the value of the fourth (unknown) resistor and (b) the voltage drop across all the resistors and prove Kirchhoff’s law.

11.	Four resistors each of 5, 20, 45 & 10 are connected in series across a d.c. supply. If the voltage across the resistors is 15, 60 & 135 volts respectively, calculate:

a.	The voltage across the 10 resistor and

b.	The supply voltage.