Unit 103: Electrical scientific principles and technologies

# Worksheet 6: Electrical supply and distribution systems (learner)

Distribution

1.	Identify the various voltages used for generation, transmission and distribution.

2.	Why is ac preferred over dc for transmission and distribution of electricity?

3.	Describe with the aid of a fully labelled diagram, the range of voltage supplies that can be derived from generation to final local distribution.

Transformers

4.	A transformer is to be wound to change an input potential of 250 volts, to an output potential of 110 volts, which is to be used for portable tools supplies. If there are 750 turns on the primary winding, how many turns will there have to be on the secondary winding.

5.	A transformer has a primary winding with 300 turns and a secondary winding of 600 turns. Calculate the secondary voltage if 250 volts is applied to the primary winding.

6.	Calculate the full load current of a transformer rated at 3300/400 volts, 60kVA.

7.	A transformer is rated at 150kVA and delivers 375 amps at full load. Calculate the output voltage of the transformer.

8.	A 200KVA 3300/240V 50Hz single phase transformer has 80 turns on the secondary winding. Assuming an ideal transformer, ie 100% efficient, calculate:

a.	primary and secondary currents on full load.

b.	the number of primary turns.

Different types of transformers

9.	A voltmeter is connected to 30 turns on the secondary winding of a VT. The primary winding of 250 turns is connected to the main supply. Calculate the supply voltage if the voltmeter reading was 60V.

10.	It is required to monitor an 11000V supply with a standard 110V voltmeter. Determine the turns ratio of the VT to perform this task.

11.	An ammeter having a full scale deflection of 5A is used to measure a line current of 300A. If the primary is wound with one turn, calculate the number of secondary turns required to give full scale deflection.

12.	Identify four other methods of generating electricity that are used in the UK.

13.	Draw a core and a shell type transformer.