Unit 103: Electrical scientific principles and technologies

# Worksheet 7: AC Theory (learner)

Resistance

Using your notes, answer the following questions:

1.	Describe with the aid of the graph the relationship between the current and voltage of an ac supply connected to a pure resistance.

2.	When an ac supply is connected to a pure resistor, state why the power produced is always ‘positive’ despite the current and the voltage going negative as well as positive.

3.	Draw the phasor diagram for voltage and current when connected to a pure resistance.

Inductance in ac circuit

Using your notes answer the following questions. Use π = 3.14.

4.	An inductance with a value of 1H is connected across a 230V 50Hz supply. Calculate:

a.	the Inductive reactance of the circuit.

b.	the current that will flow.

5.	An inductance with a value of 20mH is connected across a 400V 60Hz supply. Calculate:

a.	the Inductive reactance of the circuit.

b.	the current that will flow.

6.	An inductance with a value of 0.15H is connected across a 100V 400Hz supply. Calculate the current that will flow in the circuit.

7.	When an Inductor is connected across a 230V 50Hz supply, a current of 2.5A is recorded as flowing in the circuit. Calculate the Inductance of the coil in henrys.

8.	When an Inductor is connected across a 100V 25Hz supply, a current of 4.0A is recorded as flowing in the circuit. Calculate the Inductance of the coil in henrys.

9.	When an Inductor is connected across a 110V 325Hz supply, a current of 5.0A is recorded as flowing in the circuit. Calculate the inductance of the coil.

Capacitance in an ac circuit

Using your notes answer the following questions. Use π = 3.14.

10.	Calculate the current that will flow in the circuit, when a capacitor of 25f is connected across a 400V 60Hz supply.

11.	Calculate the current that will flow in the circuit, when a capacitor of 40f is connected across a 100V 40Hz supply.

12.	Calculate the current that will flow in the circuit, when a capacitor of 60f is connected across a 230V 50Hz supply.

13.	Calculate the size of capacitor that causes a current of 2A to flow when it is connected across a 480V 50Hz supply.

14.	Calculate the size of capacitor that causes a current of 4A to flow when it is connected across a 300V 60Hz supply.

15.	Calculate the size of capacitor that causes a current of 5A to flow when it is connected across a 400V 50Hz supply.

Impedance

Using your notes, answer the following questions.

16.	Calculate the impedance (Z) when an inductor with an inductive reactance (XL) of 30Ω is connected in a circuit with a resistance (R) of 40Ω.

17.	Calculate the power factor and the phase angle difference for the arrangement in Question 1.

18.	Calculate the impedance (Z) when a conductor with a capacitive reactance (XC) of 65Ω is connected in a circuit with a resistance (R) of 100Ω.

19.	Calculate the power factor and the phase angle difference for the arrangement in Question 3.

20.	Calculate the impedance (Z) when an inductor with an inductive reactance (XL) of 100Ω is connected in a circuit a capacitive reactance (XC) of 30Ω with a resistance (R) of 50Ω.

21.	Calculate the power factor and the phase angle difference for the arrangement in question 5.

R & L in series

Using your notes, answer the following questions. Take π as 3.14.

22.	For the arrangement in the diagram below calculate:

a.	the impedance of the circuit

b.	the phase angle of the current

c.	the current flowing in the circuit

d.	the voltage across each component the power consumed by the circuit.

continued

23.	For the arrangement in the diagram below calculate:

a.	the impedance of the circuit

b.	the phase angle of the current

c.	the current flowing in the circuit

d.	the voltage across each component the power consumed by the circuit.

continued

R and C in series

Using your notes answer the following questions. Take π as 3.14.

24.	A resistance of 80 is connected in series with a capacitor of 20F across a 230V 50Hz supply. Calculate:

a.	the impedance of the circuit

b.	the phase angle of the current

c.	the current flowing in the circuit

d.	the voltage across each component

e.	the power consumed by the circuit.

continued

25.	A resistor of 30 and a capacitor of 79.58F are connected in series across a 250V 50Hz supply. Calculate:

a.	the impedance of the circuit

b.	the phase angle of the current

c.	the current flowing in the circuit

d.	the voltage across each component

e.	the power consumed by the circuit.

continued

26.	A resistor and a capacitor are connected in series across a 50Hz ac supply and a current of 0.4139A is flowing. If the voltages across the resistor and capacitor respectively are 66.22V and 87.83V, calculate:

a.	the supply voltage

b.	the value of the resistor

c.	the value of the capacitor

d.	the power factor of the circuit.

ac power

Using your notes, answer the following questions.

27.	A particular inductive circuit produces 6kW of true power whilst producing 4kVAr of reactive power. Calculate:

a.	the apparent power

b.	the power factor.

28.	A single phase ac machine draws a current of 15A from a 400V supply. The power factor is 0.8 lagging. Calculate:

a.	the apparent power

b.	the true power

c.	the reactive power.

Power factor correction

Using your notes, answer the following questions.

29.	Explain what is meant by the term ‘power factor’.

30.	Compare methods of power factor correction.

Three phase

Using your notes, answer the following questions.

31.	A three-phase four-wire star connected transformer has a phase voltage of 190 volts and supplies a phase current of 10 amps. Calculate:

a.	the line voltage of the transformer

b.	the line current.

32.	A three-phase balanced delta connected resistive load is supplied from a transformer at a line voltage of 230 volts and draws a phase current of 11.5 amps. Calculate:

a.	the phase voltage

b.	the line current

c.	the resistance of the load.

33.	A three-phase circuit delivers 55A on the brown phase, 2A on the black phase and 97A on the grey phase. Calculate the neutral current.

34.	A three phase circuit delivers 62A on the brown phase, 29A on the black phase and 18A on the grey phase. Calculate the neutral current.